Imp/Export Smoke Testing
AUTOPIPE 10.1.0.2

CAESAR

| Test Scope | Caesar Export |
| --- | --- |
| Performed By | Shafqat Farooq |
| Date | 8-February-2016 |
| OS | Win7-64-sp1 |
| Description | Covers the smoke test scenarios executed on Caesar export module. |
| Models | QA333A,QA333B,QA333C,QA333D |
| Testing Status | FAIL |
| AutoPIPE version | 10.1.0.2 |
| Benchmark | 9.6.1.7 |

Procedure:

Caesar II Neutral

DAT

DAT

RPT

COMPARE

RPT

- Open benchmark DAT model in AutoPIPE latest version

- Analyze the model (Analyze>Analyze All)

- Generate *.rpt report (Tools>Model Input Listing)

- SaveAs Caesar II Neutral (Export DAT to *.cii)

- Open new instance of AutoPIPE and Import the *.cii (created in Step 4) and Analyze the model (Analyze>Analyze All)

- Generate *.rpt report (Tools>Model Input Listing)

- Use test case matrix sheet QA333-AP2CII-Export_0.xls for further test case execution, instructions and listing of the specific feature attributes to evaluate.

- Also, compare the RPTs generated in Step 3 and Step 6.

Relative comparisons between Model Input Listing reports (before export and after export) and, Scenarios were performed following Test Case Matrix QA333-AP2CII-Export_0 in AutoPIPE (before export and after export) for finding the point of matter. Test Case Matrix QA333-AP2CII-Export_0 has been used as an instructor for the test plan to verify that input data from an AutoPIPE DAT model is exported correctly as CII neutral file.

QA333A

“Caesar II Neutral (*.cii)” option has not been added to the selection list under the “File/Save As” menu

System Dialog:

- Project ID and Prepared by: field empty after export

- Edition: 2004, after export 2012

- Material: B313-4, after export B313-12

Pipe Data:

| Change in Pipe ids |  | Change in Pipe properties |  |
| --- | --- | --- | --- |
| Pipe ID before export | Pipe ID after export | Pipe properties before export | Pipe properties after export |
| 2"NS | PIPE1 | Schedule: STD Insul Material: AMOSITE ASBESTOS Shear Modulus: 11.0000 | Schedule: NS Insul Material: Other Shear Modulus: 11.5385 |
| 6"API5LA | PIPE3 | Schedule: 40 Insul Material: CALCIUM SILICATE | Schedule: NS Insul Material: Other |
| 10"A3331 | PIPE5 | Schedule: 40S Insul Material: FIBERGLASS | Schedule: NS Insul Material: Other |
| 16"A135A | PIPE8 | Schedule: 80; Insul Material: PERLITE | Schedule: NS; Insul Material: Other |
| 12"A105 | PIPE10 | Schedule: 120 | Schedule: NS |
| 20A335P2 | PIPE6 | Schedule: XS | Schedule: NS |
| 22A240-3 | PIPE11 | Schedule: STD Pipe Material: A240-317 Composition: Austenitic Min yield: 30000; Ultimate:75000 Long & Hoop modulus: 28.3 Shear modulus: 10.88; Density: 501 | Schedule: NS Pipe Material: CS Composition: Carbon Mini yield: 36000; Ultimate: 0.00 Long & Hoop modulus: 29.5 Shear modulus: 11.34; Density:489 |
| 8A335-P1 | PIPE13 | Schedule: 80 | Schedule: NS |
| 24A333-1 | PIPE15 | Schedule: 100; Insul Material: Perl | Schedule: NS; Insul Material: Other |
| 28A285-B | PIPE17 | Schedule: STD | Schedule: NS |
| 36A135-B | PIPE19 | Schedule: XS | Schedule: NS |

- Value of Schedule and insulation material is showing under Pipe listing Data but not under Pipe data except for 2"NS.

Operating Temperature and Pressure Data:

Component Data:

- Bends

- Tees

After export an additional Tee has been added at point B02 -> 590

- Flanges

- Valves

- Flexible joint:

- A07 ->  Y-shear, Z-shear stiffness changes from Rigid to 0.1000E+11 and Weight 10.0 to 0.00

- I07 ->  Y-shear, Z-shear stiffness changes from Rigid to 0.1000E+11 and Weight 10.0 to 0.00

Center of gravity report:

Reducer Data Listing:

Support data:

Issue in Supports and their type

User SIF:

Issue In user SIF.

Beam Point data listing:

Beam point data listing is not showing after export and beam is deleted after export.

Forces and Displacements:

Issue in Forces and Displacements values

Loads:

After export, Load Summary cannot be found

Xtra data > Additonal Weight

A02, after export additional weight at the point is replaced by a valve

Xtra data > Imposed Support Displacements

A14 and A25, imposed support displacements missing after export

QA333B

Model was successfully exported to Caesar II Neutral, but on importing *.cii file the model failed to open. An error message was delivered.

QA333C

“Caesar II Neutral (*.cii)” option has not been added to the selection list under the “File/Save As” menu

System Dialog:

- Project ID and Prepared by: field empty after export

- Edition: 2004, after export 2014

- Material: B313-4, after export B313-12

Pipe Data:

| Change in Pipe ids |  | Change in Pipe properties |  |
| --- | --- | --- | --- |
| Pipe ID before export | Pipe ID after export | Pipe properties before export | Pipe properties after export |
| 22A240-3 | PIPE1 | Schedule: STD; Pipe Material: A240-317 Composition: Austenitic Min yield: 2109.21; Ultimate:5273.02 Long Weld W factor=1 Long & Hoop modulus: 1.9897 Shear modulus: 0.7653; Density: 8025.2 Pipe Unit Wgt=131.91 | Schedule: NS; Pipe Material: CS Composition: Carbon Min yield: 2812.28; Ultimate: 0.00 Long Weld W factor is missing Long & Hoop modulus: 2.0741 Shear modulus: 0.7977; Density:7833.0 Pipe Unit Wgt=128.75 |
| 8A335-P1 | PIPE3 | Schedule: 80 | Schedule: NS |
| 24A333-1 | PIPE5 | Schedule: 100 Insul Material: Perl | Schedule: NS Insul Material: Other |
| 28A285-B | PIPE7 | Schedule: STD | Schedule: NS |
| 36A135-B | PIPE9 | Schedule: XS | Schedule: NS |

- Value of Schedule is showing under Pipe listing Data but not under Pipe data except for 22a240-3

Operating Temperature and Pressure Data:

Reducer data listing:

Bend data listing:

Center of gravity report:

Loads:

- Load summary is not shown after export

Support Data:

Similar Issue in support data for different points. Example is shown below as value of KTX and others are changed from rigid to .1000*10^11.

QA333D

“Caesar II Neutral (*.cii)” option has not been added to the selection list under the “File/Save As” menu

System Dialog:

- Project ID and Prepared by: field empty after export

- Component Library: AUTODIN, after export autopipe

Pipe Data:

| Change in Pipe ids |  | Change in Pipe properties |  |
| --- | --- | --- | --- |
| Pipe ID before export | Pipe ID after export | Pipe properties before export | Pipe properties after export |
| 2"NS | PIPE1 | Schedule: 1.4; Insul Material: Asb Pipe Material: SW-AU3 Composition: Austenitic; Min yield: 137.90 Long & Hoop modulus: 0.19493 Shear modulus: 0.07497; Density: 8025 | Schedule: NS; Insul Material: Other Pipe Material: CS Composition: Carbon; Min yield: 275.79 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 |
| 6"API5LA | PIPE3 | Schedule: 3.2; Insul Material: Calc Pipe Material: SW-MA Min yield: 206.84; Ultimate: 330.95 Long & Hoop modulus: 0.20993 Shear modulus: 0.08074 | Schedule: NS; Insul Material: Other Pipe Material: CS Min yield: 275.79; Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805 |
| 10"A3331 | PIPE5 | Schedule: 4.5; Insul Material: Fib Pipe Material: ST45.8 Min yield: 0.00; Ultimate: 320 Long & Hoop modulus: 0.21190 Shear modulus: 0.08150; Density: 7850 Pipe unit wgt=292.27 | Schedule: NS; Insul Material: Other Pipe Material: CS Min yield: 275.79; Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 Pipe unit wgt=291.63 |
| 16"A135A | PIPE8 | Schedule: 5.0; Insul Material: Perl Pipe Material: SW-UA1 Min yield: 206.84; Ultimate: 330.95 Long & Hoop modulus: 0.20593 Shear modulus: 0.07920 | Schedule: NS; Insul Material: Other Pipe Material: CS Min yield: 275.79; Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805 |
| 12"A105 | PIPE10 | Schedule: 14.2; Pipe Material: SW-AU2 Composition: Austenitic Min yield: 248.21; Ultimate: 482.63 Long & Hoop modulus: 0.19993 Shear modulus: 0.07690; Density: 8025 Pipe unit Wgt=1087.11 | Schedule: NS; Pipe Material: CS Composition: Carbon Min yield: 275.79; Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 Pipe unit Wgt=1061.10 |
| 20A335P2 | PIPE6 | Schedule: 11; Pipe Material: SW-CU2 Composition: Other; Min yield: 206.84 Ultimate: 379.21 Long & Hoop modulus: 0.12494 Shear modulus: 0.04806; Density: 8940 Pipe Unit Wgt=1505.76 | Schedule: NS; Pipe Material: CS Composition: Carbon, Min yield: 275.79 Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 Pipe Unit Wgt=1319.31 |
| 22A240-3 | PIPE11 | Schedule: 6.3; Pipe Material: SW-AU1 Composition: Austenitic Min yield: 206.84; Ultimate: 379.21 Long & Hoop modulus: 0.19993 Shear modulus: 0.07690; Density: 8025 Pipe Unit Wgt=860.57 | Schedule: NS ; Pipe Material: CS Composition: Carbon Minimum yield: 275.79; Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 Pipe Unit Wgt=839.98 |
| 8A335-P1 | PIPE13 | Schedule: 22.2; Pipe Material: SW-CU3 Composition: Other; Min yield: 206.84 Long & Hoop modulus: 0.15197 Shear modulus: 0.05845; Density: 8940 Pipe Unit Wgt=2659.82 | Schedule: NS; Pipe Material: CS Composition: Carbon; Min yield: 275.79 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 Pipe Unit Wgt=2330.46 |
| 24A333-1 | PIPE15 | Schedule: 11.0; Pipe Material: SW-AL2 Composition: Other; Min yield: 206.84 Ultimate: 330.95 Long & Hoop modulus: 0.06997 Shear modulus: 0.02691; Density: 2707 Pipe Unit Wgt=549.15 | Schedule: NS; Pipe Material: CS Composition: Carbon; Min yield: 275.79 Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 Pipe Unit Wgt=1589.02 |
| 28A285-B | PIPE17 | Schedule: 14.2; Pipe Material: ST45.8 Min yield: 186.16; Ultimate: 482.63 Long & Hoop modulus: 0.21190 Shear modulus: 0.08150; Density: 7850 Pipe Unit Wgt=2393.65 | Schedule: NS; Pipe Material: CS Min yield: 275.79; Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805; Density: 7833 Pipe Unit Wgt=2388.47 |
| 36A135-B | PIPE19 | Schedule: 16.0; Pipe Material: SW-CM1 Min yield: 241.32; Ultimate: 482.63 Long & Hoop modulus: 0.20993 Shear modulus: 0.08074 | Schedule: NS; Pipe Material: CS Min yield: 275.79; Ultimate: 0.00 Long & Hoop modulus: 0.20293 Shear modulus: 0.07805 |

Coordinates Data listing:

Some differences were observed in X, Y, Z coordinates.

Operating Temperature and Pressure Data:

Forces and Displacements:

Component Data:

- Bends

- Multiport component > Tee: For all the Tees the type of tee and material has changed

- After export an additional Tee is found at B02 -> 590

Flanges

There are 13 flanges before exporting and 9 flanges after exporting. Also there is difference in values.

Valves

Issue in valves and their placement. Also there are 11 valves before exporting and 16 valves after exporting.

Flexible Joints

A08 -> 105, weight changes from 4.54 to 0.00

Beam Point data listing:

Beam point data listing is not showing after export and beam is deleted after export

Support data:

- Issue in Supports and their type

- Extra supports are present after export

Reducer data listing:

Rigid pipe data listing:

After export, rigid pipe data cannot be found

Center of gravity report:

Loads:

After export, Loads Summary cannot be found

Xtra data > Additional Weight

A02, after export additional weight at the point is replaced by a valve

Xtra data > Imposed Support Displacements

A14 and A25, imposed support displacements missing after export