Imp/Export Smoke Testing
AUTOPIPE 11.0.0.15

PXF

| Test Scope | PXF Export |
| --- | --- |
| Performed By | Shafqat Farooq |
| Date | 29th November 2016 |
| Machine | WIN-64-SP1 |
| Description | Covers the smoke test scenarios executed on PXF Import module. |
| Models | QA325A, QA325B |
| Testing Status | Issues in supports |
| AutoPIPE version | 11.0.0.15 |
| Benchmark | 9.6.0.19 |
| AutoPlant | 8.11.13.620 |

Procedure:

- Open DAT model in AutoPIPE latest version.

- Analyze the model (Analyze>Analyze All)

- Generate *.rpt report (Tools>Model Input Listing)

- Export the model to AutoPLANT(*.pxf)

- Import/Open the *.pxf file in AutoPLANT

- Examine the following features in AutoPLANT

Feature Tested:

- Export PXF file and check that the export is done without errors

- Check vertical axis

- Check result units.

- Open QA325A model with English units in AutoPLANT

- Open QA325B model with Metric units in AutoPLANT

- Compare *.pxf in AutoPLANT  and *.dat in AutoPIPE with geometric perspective

Result:  (Benchmark: 9.6.0.19)

| Comparison | QA325A | QA325B |
| --- | --- | --- |
| Geometry(length and coordinates) | OK | OK |
| Pipe run points starting or ending a segment | OK | OK |
| Pipe run points not connected to components | OK | OK |
| Bend Tangent intersection points (CP of bend in AutoPLANT) | OK | OK |
| Tee points | OK | OK |
| Flange | Considered OK | Considered OK |
| Reducer | OK | OK |
| Valve | OK | OK |
| Nozzle | OK | OK |
| Flexible joint | OK | OK |
| Anchors | Considered OK | Considered OK |
| Supports | Not OK | Not OK |

Same issues were found for both the models i.e. QA325A and QA325B

Issues in Supports:

- Point A03N (X=10 Y=23 Z=0)  Line Stop Support is missing at the point of a bend  [Not OK]

- Between Point A03 and A04, Shoe Channel Support was found in AutoPLANT at Pipe point: X=3048 Y=6858 Z=0                                                                                                                               [Not OK]

- Point A0F (X=9.75 Y=23 Z=8) Constant Support with 2 hangers has been replaced by Spring Support

(Considered OK, after viewing HELP DESK and PXFout.map file for Mapping)

- Point A12 (X=8 Y=25 Z=-1) V Stop Support missing at the point in AutoPLANT                      [Not OK]

- Point A16 (X=8 Y=27 Z=8)            [Not OK]

- Spring Hanger Support additionally found at the point in AutoPLANT

- Point A17 (X=5 Y=27 Z=8) Tie Link Support is replaced by Base Support. In AutoPIPE a beam section is drawn from point F17 to  F20  (Considered OK, after viewing HELP DESK and PXFout.map file  for Mapping)

Issues in Flanges: (Considered OK, after viewing HELP DESK for Mapping)

- Point A09 (X=8 Y=23 Z=5)  Slip On Flange is missing at the point in AutoPLANT

- Point A19N (X=2.25 Y=27 Z=8) and Point A19F (X=2 Y=27.25 Z=8) No flanges found at these two points in AutoPLANT

Issues in Anchors: (Considered OK, after viewing HELP DESK and PXFout.map for Mapping)

- Point A00 (X=10 Y=10 Z=0) Rigid anchor is replaced by Angle Guide Support

- Point A21 (X=2 Y=30 Z=8) Rigid Anchor replaced by Angle Rigid Support

- Point B05 (X=14 Y=25 Z=-1) Flexible Anchor has been replaced by Angle Guide Support